📊 COMPREHENSIVE PROJECT STATUS REPORT

Project: AVA - Autonomous Verification Agent for RISC-V
Target Scope: RV32IM → RV32IMAC → Full SOTA Verification
Report Date: February 17, 2026

EXECUTIVE SUMMARY

Current State: Framework and orchestration complete (40%), but verification engine non-functional (0% actual RTL verification capability).

Critical Gap: No real simulation backend, no ISS integration, no coverage extraction.

Strategic Decision: ✅ RV32IM first (Integer + Multiply/Divide), then expand to RV32IMAC (add Atomics + Compressed).

Why RV32IM?

- M extension critical for real workloads (multipliers are complex microarchitecture)

- Smaller scope than full IMAC (defer atomics and compressed until core loop works)

- Compliance suite has excellent RV32IM coverage

- Differential testing easier (Spike supports IM perfectly)

📋 PHASE-BY-PHASE STATUS MATRIX

| Component | Status | Completeness | Blocker? | Priority |
| --- | --- | --- | --- | --- |
| Real RTL Core (DUT) | ❌ Missing | 0% | 🔴 YES | P0 |
| Verilator Integration | ❌ Missing | 0% | 🔴 YES | P0 |
| Spike ISS Integration | ❌ Missing | 0% | 🔴 YES | P0 |
| Commit Log Comparator | ❌ Missing | 0% | 🔴 YES | P0 |
| VCD Coverage Parser | ❌ Missing | 0% | 🔴 YES | P0 |
| Test Harness (Cocotb) | ⚠️ Template only | 15% | 🔴 YES | P0 |
| RISC-V Compliance Tests | ❌ Not integrated | 0% | 🟡 MEDIUM | P1 |

PHASE 0: FOUNDATIONS (CRITICAL - MUST COMPLETE FIRST)

STATUS: ❌ BLOCKED - Cannot verify anything until these exist

Immediate Action Required:

- Select/obtain RV32IM RTL core (open-source or write minimal)

- Build Verilator simulation wrapper

- Integrate Spike ISS with identical program image

- Create commit-log format and comparator script

PHASE 1: CORE INTELLIGENCE (40% Complete)

| Component | Status | Completeness | Notes |
| --- | --- | --- | --- |
| AVA Core Engine | ⚠️ Scaffold exists | 40% | Orchestration works, backends mocked |
| Semantic Analysis | ⚠️ Partial | 50% | LLM + rule-based RTL parsing functional |
| Testbench Generation | ⚠️ Template only | 25% | Generates Cocotb/UVM skeletons, no stimulus |
| Spike ISS Class | ⚠️ Mock | 10% | Structure exists, run_tandem() is placeholder |
| Coverage Director | ⚠️ Mock | 30% | RL framework present, no real coverage input |
| RAG System | ❌ Not started | 0% | FAISS index planned, not implemented |
| LLM Integration | ✅ Working | 80% | Ollama integration functional |

FILES COMPLETED:

- ✅ core/ava_engine.py - Main orchestration (mocked backends)

- ✅ core/semantic_analyzer.py - RTL parsing

- ✅ core/testbench_generator.py - Template generation

- ✅ core/spike_iss.py - ISS wrapper (placeholder)

- ✅ core/coverage_director.py - Adaptive test generation (needs real data)

GAPS:

- _simulate_rtl() returns hardcoded values

- _simulate_iss() returns hardcoded values

- _calculate_coverage() uses fake percentages

- No actual VCD parsing

- No real commit-log comparison

PHASE 2: RED TEAM ADVERSARIAL AGENTS (25% Complete)

| Component | Status | Completeness | Notes |
| --- | --- | --- | --- |
| ConflictAgent | ⚠️ Logic complete | 60% | Attack patterns defined, needs DUT hooks |
| SpeculationAgent | ⚠️ Logic complete | 60% | Spectre attacks coded, needs RTL speculation |
| PowerAgent | ⚠️ Logic complete | 50% | Power virus patterns ready, no power measurement |
| DependencyGraph | ✅ Functional | 85% | Cycle detection works, needs NoC transaction feed |
| MultiAgentManager | ⚠️ Orchestration ready | 40% | Swarm coordination works, agents are mocks |

FILES COMPLETED:

- ✅ redteam/agents/conflict_agent.py - Cache coherency attacks

- ✅ redteam/agents/speculation_agent.py - Spectre/Meltdown patterns

- ✅ redteam/agents/power_agent.py - Power virus generation

- ✅ redteam/graph/dependency_graph.py - Deadlock detection

- ✅ core/multi_agent_manager.py - Swarm orchestrator

GAPS:

- Agents have no connection to actual RTL signals

- No cocotb monitor hooks

- No real power/thermal data source

- Cannot trigger actual bugs yet (need real RTL first)

PHASE 3: API & INTEGRATION (10% Complete)

| Component | Status | Completeness | Notes |
| --- | --- | --- | --- |
| FastAPI Server | ⚠️ Endpoints defined | 15% | Routes exist, no backend integration |
| WebSocket Streaming | ❌ Not started | 0% | For real-time verification updates |
| Result Storage | ⚠️ Basic JSON | 30% | Saves to files, no database |
| Authentication | ❌ Not started | 0% | Planned for production |
| Rate Limiting | ❌ Not started | 0% | Planned for production |

FILES COMPLETED:

- ✅ api/main.py - FastAPI skeleton (mocked responses)

GAPS:

- No connection to AVA engine

- No streaming verification results

- No proper error handling

- No production hardening

PHASE 4: KUBERNETES DEPLOYMENT (0% Complete)

| Component | Status | Completeness | Priority |
| --- | --- | --- | --- |
| Docker Containerization | ❌ Not started | 0% | P2 |
| K8s Manifests | ❌ Not started | 0% | P2 |
| Helm Charts | ❌ Not started | 0% | P3 |
| CI/CD Pipeline | ❌ Not started | 0% | P2 |
| Monitoring/Logging | ❌ Not started | 0% | P2 |

STATUS: Deferred until Phases 0-3 functional

🎯 DETAILED ROADMAP: RV32IM → RV32IMAC → FULL VERIFICATION

MILESTONE 1: RV32IM TRUTH ENGINE (8-10 weeks)

Goal: Pass RISC-V RV32IM compliance suite with differential testing

Week 1-2: RTL Acquisition & Setup

- Select RV32IM core: Options:

  - PicoRV32 (simple, 2-stage, well-tested)

  - Ibex (lowRISC, 2-stage, security features)

  - VexRiscv (configurable, good docs)

  - Write minimal 5-stage RV32IM core (educational)

- Set up Verilator build: Compile RTL to C++ model

- Create test harness: Basic Cocotb wrapper with clock/reset

- Memory model: Simple RAM (no caches yet)

- Boot ROM: Minimal bootloader to load test programs

Deliverable: RTL runs "hello world" program to completion

Week 3-4: ISS Integration & Differential Testing

- Build Spike wrapper:

  - Install RISC-V Spike ISS

  - Create Python wrapper (spike_wrapper.py)

  - Load same ELF binary as RTL

  - Configure for RV32IM (no A, no C, M-mode only)

- Commit log format:

json

{"cycle": 123, "pc": "0x80000000", "instr": "0x00508093", "x1": "0x12345678", "x2": "0xabcdef00", ...}

- Comparator script: Diff commit logs, report first mismatch

- Run differential tests: Start with simple programs (add, loops, jumps)

Deliverable: 10 hand-written tests pass differential verification

Week 5-6: RISC-V Compliance Suite

- Download compliance tests: riscv-arch-test repository

- Filter RV32IM tests: ~150 tests for I + M extensions

- Create test runner: Batch script to run all compliance tests

- Fix mismatches: Debug RTL vs Spike differences

- Coverage measurement: Parse VCD for line/toggle coverage

Deliverable: 100% RV32IM compliance tests pass

Week 7-8: Random Instruction Generation

- Build constrained random generator:

  - Legal RV32IM instructions only

  - Valid register operands

  - Aligned PC jumps

  - No illegal instructions

- Long-run tests: 10K, 100K, 1M instruction streams

- Detect non-determinism: Multiple runs with same seed must match

- Bug discovery: Find corner cases compliance missed

Deliverable: 1M instruction random test passes, reproducible

Week 9-10: M Extension Deep Dive

- Multiply/divide stress:

  - Edge cases: 0×N, MAX×MAX, MIN×MIN

  - Divide by zero (should trap)

  - Multi-cycle operation timing

- Performance testing:

  - Measure multiply latency

  - Measure divide latency

  - Check pipeline stalls

- Coverage closure: Target 95%+ toggle coverage on multiplier

Deliverable: M extension fully verified, production-grade

MILESTONE 2: RV32IMAC (4-6 weeks)

Add Atomics (A) and Compressed (C) extensions

Week 11-12: A Extension - Atomics

- AMO instructions: LR/SC, AMOSWAP, AMOADD, AMOAND, AMOOR, AMOXOR, AMOMIN, AMOMAX

- Memory ordering: Acquire/release semantics

- Multi-core prep: Single-core atomics first (no contention)

- Compliance tests: RV32A tests

- Directed tests: LR/SC success/fail scenarios

Deliverable: RV32IMA compliance passes

Week 13-14: C Extension - Compressed Instructions

- 16-bit encoding: C.ADD, C.ADDI, C.LW, C.SW, C.J, C.JALR, C.BEQZ, etc.

- Decompression logic: Expand C.* to 32-bit equivalents

- Mixed 16/32-bit code: Test transitions

- Compliance tests: RV32C tests

- Code density measurement: Compare compressed vs uncompressed sizes

Deliverable: RV32IMAC full compliance

Week 15-16: Coverage & Performance

- Functional coverage: Create covergroups for all instruction types

- Branch coverage: All conditional paths taken

- Corner case coverage: Edge values, exceptions, traps

- Performance baseline: IPC, branch prediction accuracy

- Power estimation: Cycle-accurate power model (if available)

Deliverable: 95%+ coverage, industrial-grade RV32IMAC verification

MILESTONE 3: PRIVILEGED ARCHITECTURE (6-8 weeks)

Add M/S/U modes, CSRs, traps, interrupts

CSR Implementation

- Machine mode CSRs: mstatus, mie, mip, mtvec, mscratch, mepc, mcause, mtval

- Supervisor mode CSRs: sstatus, sie, sip, stvec, sscratch, sepc, scause, stval

- Counter CSRs: mcycle, minstret, time

- Trap entry/return: mret, sret

- PMP (Physical Memory Protection)

Trap Testing

- Illegal instruction

- Misaligned access

- Instruction page fault

- Load/store page fault

- Breakpoint

- Environment call (ecall)

- External interrupt

- Timer interrupt

Deliverable: OS-ready core (can boot Linux)

MILESTONE 4: VIRTUAL MEMORY (4-6 weeks)

Sv32/Sv39 page tables, TLB, page faults

- Page table walker

- TLB (I-TLB, D-TLB)

- Page fault handling

- Accessed/dirty bits

- Superpages

- Linux boot test

Deliverable: Linux boots to login prompt

MILESTONE 5: MULTICORE & COHERENCE (8-12 weeks)

2-4 core SMP, cache coherence, memory model

- Dual-core configuration

- MESI/MOESI coherence protocol

- RVWMO memory model validation

- Litmus tests (load buffering, store buffering, IRIW, etc.)

- Cross-core interrupts

- Barrier instructions (FENCE, FENCE.I)

Deliverable: SMP Linux boots, litmus tests pass

MILESTONE 6: SECURITY VERIFICATION (6-8 weeks)

Spectre/Meltdown, side-channels, fault injection

- Red team agent integration with real RTL

- Spectre v1/v2/v4 tests

- Cache timing attack detection

- Branch predictor poisoning tests

- Power side-channel analysis

- Fault injection simulation

Deliverable: Security certification report

MILESTONE 7: PERFORMANCE & POWER (4-6 weeks)

IPC optimization, power modeling, thermal analysis

- Performance counters

- Pipeline visualization

- Bottleneck analysis

- Power virus detection

- Thermal hotspot identification

- DVFS verification

Deliverable: Performance/power characterization report

MILESTONE 8: ADVANCED FEATURES (8-12 weeks)

OoO execution, vector extension, formal verification

- Out-of-order pipeline verification

- RVV (Vector) extension

- Formal property checking

- Model checking

- Theorem proving (RISC-V spec in Coq/Isabelle)

Deliverable: SOTA verification capability

📊 VERIFICATION COVERAGE MATRIX

ISA Coverage (RV32IM → RV32IMAC)

| Extension | Instructions | Compliance Tests | Random Tests | Differential | Coverage Target |
| --- | --- | --- | --- | --- | --- |
| RV32I | 40 | ✅ Required | ✅ Required | ✅ Required | 100% |
| M | 8 | ✅ Required | ✅ Required | ✅ Required | 100% |
| A | 11 | 🔲 Future | 🔲 Future | 🔲 Future | 100% |
| C | 35 | 🔲 Future | 🔲 Future | 🔲 Future | 100% |

Microarchitecture Coverage

| Component | Line Cov | Branch Cov | Toggle Cov | Functional Cov | Status |
| --- | --- | --- | --- | --- | --- |
| Instruction Fetch | 95%+ | 90%+ | 95%+ | 90%+ | 🔲 Pending |
| Decoder | 100% | 100% | 95%+ | 100% | 🔲 Pending |
| ALU | 100% | 100% | 95%+ | 100% | 🔲 Pending |
| Multiplier | 95%+ | 90%+ | 95%+ | 95%+ | 🔲 Pending |
| Divider | 95%+ | 90%+ | 95%+ | 95%+ | 🔲 Pending |
| Load/Store Unit | 95%+ | 90%+ | 95%+ | 90%+ | 🔲 Pending |
| Branch Unit | 95%+ | 95%+ | 95%+ | 95%+ | 🔲 Pending |
| CSR Unit | 95%+ | 90%+ | 90%+ | 90%+ | 🔲 Pending |
| Pipeline Control | 95%+ | 90%+ | 90%+ | 90%+ | 🔲 Pending |

Security & Reliability Coverage

| Category | Tests Required | Status | Target |
| --- | --- | --- | --- |
| Spectre v1 | 20+ | ❌ | 100% detection |
| Spectre v2 | 15+ | ❌ | 100% detection |
| Side-channel | 30+ | ❌ | No leaks |
| Fault injection | 100+ | ❌ | No silent failures |
| Power analysis | 50+ | ❌ | No DPA leaks |

📂 DATA REQUIREMENTS

Input Data Needed

| Data Type | Source | Format | Size | Priority |
| --- | --- | --- | --- | --- |
| RTL Core | Open-source or custom | Verilog/SystemVerilog | ~10K lines | P0 |
| RISC-V Compliance Tests | riscv-arch-test repo | ELF binaries | ~150 tests | P0 |
| Spike ISS | riscv-isa-sim repo | Binary + library | ~50MB | P0 |
| Toolchain | riscv-gnu-toolchain | GCC/Binutils | ~1GB | P0 |
| Litmus Tests | herd7 suite | Assembly | ~500 tests | P2 |
| Microbenchmarks | Embench, CoreMark | Source | ~100KB | P2 |

Output Data Generated

| Data Type | Purpose | Format | Retention |
| --- | --- | --- | --- |
| Commit Logs | Differential testing | JSONL | 7 days |
| VCD Waveforms | Debug & coverage | VCD/FST | 30 days |
| Coverage Reports | Metrics tracking | HTML/JSON | Forever |
| Bug Reports | Issue tracking | JSON | Forever |
| Performance Metrics | Optimization | CSV | Forever |
| Security Reports | Certification | PDF/JSON | Forever |

🚧 KNOWN GAPS & TECHNICAL DEBT

Critical Gaps (Must Fix)

- No real simulation backend - Everything is mocked

- No ISS integration - Cannot do differential testing

- No coverage extraction - VCD parser missing

- No test corpus - Need compliance tests integrated

- No commit log format - Cannot compare RTL vs ISS

Technical Debt

- Error handling - Many functions lack proper exception handling

- Logging - Inconsistent log levels and verbosity

- Configuration - Hardcoded paths and parameters

- Documentation - API docs incomplete

- Testing - No unit tests for AVA itself

💰 COST & EFFORT ESTIMATE

Phase 0-2 (RV32IM → RV32IMAC)

- Timeline: 14-16 weeks

- Effort: 1 engineer full-time

- Compute: Minimal (laptop sufficient)

- Storage: ~100GB for waveforms/logs

Phase 3-5 (Privileged + VM + Multicore)

- Timeline: 18-26 weeks

- Effort: 1-2 engineers

- Compute: Workstation (32+ cores for parallel sims)

- Storage: ~500GB

Phase 6-8 (Security + Performance + Advanced)

- Timeline: 18-24 weeks

- Effort: 2-3 engineers

- Compute: Small cluster (for formal methods)

- Storage: ~1TB

Total: 50-66 weeks, 1-3 engineers, ~$250K-$500K (vs $20M for commercial tools)

🎯 SUCCESS CRITERIA

Milestone 1 Success (RV32IM)

- ✅ 100% RISC-V RV32IM compliance tests pass

- ✅ 1M+ instruction random tests pass without mismatches

- ✅ 95%+ toggle coverage on all datapath components

- ✅ Multiplier/divider fully characterized

- ✅ Differential testing infrastructure robust

Milestone 2 Success (RV32IMAC)

- ✅ 100% RISC-V RV32IMAC compliance tests pass

- ✅ Atomics work correctly (LR/SC never deadlock)

- ✅ Compressed instructions decode correctly

- ✅ Mixed 16/32-bit code executes properly

Production Ready

- ✅ 95%+ functional coverage

- ✅ 0 critical bugs

- ✅ Security certified (no Spectre/side-channels)

- ✅ OS boots (Linux/FreeRTOS)

- ✅ Performance characterized

- ✅ Formal proofs for critical paths

📋 NEXT IMMEDIATE ACTIONS (This Week)

Priority 0 (Do First)

- Select RTL core: Download PicoRV32 or Ibex

- Install Verilator: sudo apt install verilator

- Install Spike: Build from source

- Install RISC-V toolchain: riscv32-unknown-elf-gcc

- Clone compliance tests: git clone riscv-arch-test

Priority 1 (Do Next)

- Write Verilator wrapper: Basic C++ testbench

- Write Spike wrapper: Python script to drive ISS

- Define commit log format: JSON schema

- Write comparator: Python script to diff logs

- Run first differential test: Hello world program

📝 RECOMMENDATIONS

Strategic

- ✅ RV32IM first is the right choice - solid foundation

- ✅ Differential testing is your north star - never compromise

- ⚠️ Don't add features until truth engine works

- ⚠️ Defer agents until you have real RTL signals to attack

- ⚠️ Defer Kubernetes until core verification works

Tactical

- Use PicoRV32 - simplest correct RV32IM implementation

- Start with Verilator - fastest simulation

- Batch compliance tests - automate from day 1

- VCD only for debug - too slow for regression

- Commit logs are truth - invest in good format design

Process

- Daily regression - all passing tests must keep passing

- Coverage ratchet - never allow coverage to decrease

- Bug triage - categorize by severity (functional vs performance)

- Weekly reviews - track progress against milestones

- Document everything - future you will thank present you

🎓 LEARNING RESOURCES NEEDED

- RISC-V Spec: Read Volume I (User-Level ISA) cover-to-cover

- Spike Source Code: Understand golden model implementation

- Verilator Manual: Learn VCD generation and coverage

- Cocotb Docs: Master async testbench patterns

- Computer Architecture: Hennessy & Patterson (background)